|  | PERFORMANCE KNOWLEDGE & SKILL "ON THE JOB" VERIFICATION |  |
| --- | --- | --- |
|  | FALL PROTECTION SKILLS CHECKLIST |  |
|  | Objectives: |  |
|  |  | * Identify the different types of fall protection |
|  |  | * Demonstrate understanding through hands on exercise |
|  |  | * Demonstrate knowledge of attach points through discussion |
|  |  | * Demonstrate the proper procedures for inspecting, donning, and using full body harnesses and lanyards |
| WARNING! YOU MUST NOT PERFORM ELEVATED WORK ON LOCATIONS WHERE THE PROPER EQUIPMENT OR |  |  |
|  |  | TRAINED PERSONNEL ARE NOT AVAILABLE |
|  | KNOWLEDGE & SKILLS YOU MUST DEMONSTRATE & PERFORM |  |
| 1. | BODY HARNESS INSPECTION |  |
|  | INSPECT THE FOLLOWING: |  |
|  |  | * Labels - to insure they are secure and easy to read. |
|  |  | * Belts and straps - for fraying, broken fibers, pulled stitches, cuts, discoloration, and charring from heat / chemical damage. |
|  |  | * Dee Rings - for distortion, cracks, brakes, and rough or sharp edges. |
|  |  | * Tongue buckles - sharp edges, corrosion, burrs, cracks, dents, or distortion. |
|  |  | * Tongue buckle grommets - rough or sharp edges, corrosion, burrs, cracks, dents, or distortion. |
|  |  | * Friction buckles - sharp edges, corrosion, burrs, cracks, dents, or distortion. |
| 2. | LANYARD INSPECTION |  |
|  | INSPECT THE FOLLOWING: |  |
|  |  | * Labels - to insure they are secure and easy to read. |
|  |  | * Hardware (Snaps) - for hook and eye distortions, cracks, corrosion, or pitted surfaces. Inspect latch and keeper spring / lock. |
|  |  | * Shock Absorber - to see that it is intact and free of cracks or splits. |
|  |  | * Web Lanyard - for tears, cuts, fraying, knots, swelling, discoloration, and charring from heat / chemical damage. |
|  |  | * Steel Lanyard - for cuts, frayed areas, or unusual wear patterns. |
|  |  | * Rope Lanyard - for fuzzy, worn, broken, change of diameter, or cut fibers. |
| 3. | TIE - OFF STRAP INSPECTION |  |
|  | INSPECT THE FOLLOWING: |  |
|  |  | * Labels - to insure they are secure and easy to read. |
|  |  | * Web Lanyard - for tears, cuts, fraying, knots, swelling, discoloration, and charring from heat / chemical damage. |
|  |  | * Dee Rings - for distortion, cracks, brakes, and rough or sharp edges. |
| 4. | SELF RETRACTING LIFELINE INSPECTION |  |
|  | INSPECT THE FOLLOWING: |  |
|  |  | * Labels - to insure they are secure and easy to read. |
|  |  | * Lifeline - rip-stitch impact indicator for indications that it has been impact loaded. |
|  |  | * Housing - for cracks, dents, or distortions. |
|  |  | * Swivel Eye - for cracks, distortion, and wearing. |
|  |  | * Connecting Hook - for hook and eye distortions, cracks, corrosion, or pitted surfaces. Inspect latch and keeper spring / lock. |
| 5. | ATTACH POINTS |  |
|  | DISCUSS THE FOLLOWING: |  |
|  |  | * Anchorage - must be capable of supporting 5,000 pounds per attached worker. |
|  |  | * Snap Hooks - must be locking type and never hook a snap hook into another snap hook. |
|  |  | * Lanyards - hook to an anchorage point. Never hook back into your lanyard. |
| CERTIFICATION OF SATISFACTORY KNOWLEDGE & SKILLS DIRECTLY OBSERVED: |  |  |
| My signature below certifies the below named personnel have performed the above tasks and are capable of protecting themselves and |  |  |
| others in the use of fall protection equipment. |  |  |
| CERTIFYING SIGNATURE & DATE |  |  |
| NAME(S) & EMPLOYEE NUMBER OF PERSONNEL CERTIFIED (LIST OR ATTACHED ROSTER) |  |  |